NM1520RD2AE腳位規畫

| Pin No. | Pin Name | Discribtion | Pin definition |  |
| --- | --- | --- | --- | --- |
| 1 | PVSS | PLL GROUND: PLL Ground potential. |  |  |
| 2 | P9.7 | General purpose digital I/O pin | Mcc | Reserve |
|  | SPI1_SS | SPI1 slave select pin |  |  |
| 3 | P3.5 | General purpose digital I/O pin | SCL | IIC |
|  | TM1 | Timer1 external clock |  |  |
|  | ISC0_SCL | I2C0 clock output pin |  |  |
| 4 | P3.4 | General purpose digital I/O pin | SDA | IIC |
|  | TM0 | Timer0 external clock |  |  |
|  | ISC0_SDA | I2C0 data input/output pin |  |  |
| 5 | P1.7 | General purpose digital I/O pin | Fault |  |
|  | PWM1_BREAK0 | Brake input pin0 of PWM Unit 1 |  |  |
| 6 | LDO_CAP | LDO: LDO output pin Note: It needs to be connected with a 1uF capacitor. |  |  |
| 7 | VDD | POWER SUPPLY: Supply voltage Digital VDD for operation. |  |  |
| 8 | Vss | GROUND: Digital Ground potential. |  |  |
| 9 | PA.1 | General purpose digital I/O pin | KP_RXD |  |
|  | UART1_RXD | Data Receiver input pin for UART1 |  |  |
|  | I2C0_SCL | I2C0 clock output pin |  |  |
| 10 | PA.0 | General purpose digital I/O pin | KP_TXD |  |
|  | UART1_TXD | Data transmitter output pin for UART1 |  |  |
|  | I2C0_SDA | I2C0 data input/output pin |  |  |
| 11 | P5.6 | General purpose digital I/O pin | DcBreak | Reserve |
|  | BPWM0_CH0 |  |  |  |
| 12 | P1.6 | General purpose digital I/O pin | Esp | Reserve |
|  | EPWM0_BREAK0 | Brake input pin 0 of EPWM Unit 0 |  |  |
| 13 | P1.5 | General purpose digital I/O pin | Wn |  |
|  | EPWM1_CH5 | PWM5 output of EPWM Unit 1 |  |  |
| 14 | P1.4 | General purpose digital I/O pin | Wp |  |
|  | EPWM1_CH4 | PWM4 output of EPWM Unit 1 |  |  |
| 15 | P1.3 | General purpose digital I/O pin | Vn |  |
|  | EPWM1_CH3 | PWM3 output of EPWM Unit 1 |  |  |
| 16 | P1.2 | General purpose digital I/O pin | Vp |  |
|  | EPWM1_CH2 | PWM2 output of EPWM Unit 1 |  |  |
| 17 | P3.3 | General purpose digital I/O pin | NC |  |
|  | INT1 | External Interrupt 1 input pin |  |  |
| 18 | P1.1 | General purpose digital I/O pin | Un |  |
|  | EPWM1_CH1 | PWM1 output of EPWM Unit 1 |  |  |

| 19 | P1.0 | General purpose digital I/O pin | Up |  |
| --- | --- | --- | --- | --- |
|  | EPWM1_CH0 | PWM0 output of EPWM Unit 1 |  |  |
| 20 | P3.0 | General purpose digital I/O pin | Rs485_Rxd | Reserve |
|  | UART0_RXD | Data Receiver input pin for UART0 |  |  |
|  | CLKO | Clock output |  |  |
| 21 | P3.1 | General purpose digital I/O pin | Rs485_Txd | Reserve |
|  | UART0_TXD | Data transmitter output pin for UART0 |  |  |
| 22 | Vss |  |  |  |
| 23 | P5.0 | General purpose digital I/O pin | DA_DAT | Debug DA |
|  | SPI0_MOSI |  |  |  |
|  | UARTn_RTS/I2C_SCL |  |  |  |
| 24 | P5.1 | General purpose digital I/O pin | Rs485_De | Reserve |
|  | SPI0_MISO |  |  |  |
|  | UARTn_CTS/I2C_SDA |  |  |  |
| 25 | P2.7 | General purpose digital I/O pin | DA_CLK | Debug DA |
|  | SPI0_CLK |  |  |  |
|  | UART1_RTS |  |  |  |
| 26 | P2.6 |  | DA_CS | Debug DA |
|  | QEI0_Z |  |  |  |
|  | SPI0_SS |  |  |  |
| 27 | P2.5 |  | EnCoder_B/ Hv | Reserve |
|  | QEI0_B |  |  |  |
|  | CAN0_TXD |  |  |  |
| 28 | P2.4 |  | EnCoder_A/ Hu | Reserve |
|  | QEI0_A |  |  |  |
|  | CAN0_RXD |  |  |  |
| 29 | P0.7/STADC |  | Hw |  |
| 30 | P0.6/EPWM0BK1 |  | FANC | Reserve |
| 31 | P0.5 |  | Up/down | Reserve |
|  | EPWM_CH5 |  |  |  |
| 32 | P0.4 |  | Incline_Run | Reserve |
|  | EPWM_CH4 |  |  |  |
| 33 | P0.3 |  |  |  |
|  | EPWM_CH3 |  |  |  |
| 34 | P0.2 |  |  |  |
|  | EPWM_CH2 |  |  |  |
| 35 | P0.1 |  |  |  |
|  | EPWM_CH1 |  |  |  |

| 36 | P0.0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | EPWM_CH0 |  |  |  |
| 37 | P6.6 |  |  |  |
|  | ADC0_CH6 |  |  |  |
| 38 | P6.5 |  |  |  |
|  | ADC0_CH5 |  |  |  |
| 39 | P6.4 |  | NTC | Reserve |
|  | ADC0_CH4 |  |  |  |
| 40 | P6.3 |  | Vbus |  |
|  | ADC0_CH3 |  |  |  |
| 41 | P6.2 |  | Vgrid_R |  |
|  | ADC0_CH2 |  |  |  |
| 42 | P6.1 |  | Is | I sum |
|  | ADC0_CH1 |  |  |  |
| 43 | P6.0 |  | Iu | Iu current |
|  | ADC0_CH0 |  |  |  |
| 44 | P8.2 |  |  |  |
| 45 | P8.1 |  |  |  |
| 46 | P8.0 |  |  |  |
| 47 | AVss |  |  |  |
| 48 | AV_DD |  |  |  |
| 49 | P7.6 |  |  |  |
|  | ADC1_CH6 |  |  |  |
| 50 | P7.5 |  |  |  |
|  | ADC1_CH5 |  |  |  |
| 51 | P7.4 |  |  |  |
|  | ADC1_CH4 |  |  |  |
| 52 | P7.3 |  |  |  |
|  | ADC1_CH3 |  |  |  |
| 53 | P7.2 |  | Vgrid_T |  |
|  | ADC1_CH2 |  |  |  |
| 54 | P7.1 |  | Vgrid_S |  |
|  | ADC1_CH1 |  |  |  |
| 55 | P7.0 |  | Iv | Iv current |
|  | ADC1_CH0 |  |  |  |
| 56 | P9.0 |  |  |  |
| 57 | P9.1 |  |  |  |
| 58 | P9.2 |  | KP_DE |  |
| 59 | Vdd |  |  |  |

| 60 | RESET |  | Reset |  |
| --- | --- | --- | --- | --- |
| 61 | XT1_OUT |  | XT1_OUT |  |
| 62 | XT1_IN |  | XT1_IN |  |
| 63 | ICE_DAT |  | ICE_DAT |  |
| 64 | ICE_CLK |  | ICE_CLK |  |